The goal of this FURI project is to understand and quantify the affect that the properties of a particle  have on their discharge flow rate through a hopper. This project is of value because of the industrial importance of particles which are commonly stored and discharged by hoppers. Knowing how a particle will behave while being discharged from a hopper, will help engineers avoid problems dealing with content uniformity of the discharge, particles adhering to the sides of the hopper, and  particle clogging the hoppers outlet, stopping the flow.

I am obtaining plots of mass of the bead’s verses discharge time for various bead size as they are discharged from a hopper into a container on a balance. This balance records the mass of the beads at half second intervals. In this project I will focus on using three sizes of glass silica beads with size diameters of 0.6 mm, 2 mm and 4 mm.  The size of the hopper will also be varied using cylindrical hoppers of 9.5 cm, and 11 cm in diameters. I will then characterize each bead size using Atomic Force Microcopy (AFM), to obtain their corresponding surface roughness. The surfaces of the hoppers will also be characterized using AFM. This will then be the basis of my future work of changing the roughness of the beads and hoppers through chemical etching.

This project has given me many learning opportunities as well as challenges to overcome. My favorite leaning opportunity was when I received training on an AFM which is tool used to characterize the surfaces of particles. This will allow me to be able to apply this technique to my research, to quantitively characterizing the roughness of the glass beads. Developing this skill in the AFM has widen my skill sets and has given me another valuable skill to add to my resume. Learning how to quantify and study particle properties has supplemented gaps in my education, because particulate processes are not heavily touched on, in chemical engineering curriculum. This research has taught me to seek out solutions to problems even if the solution is not obvious. The first problem I encountered was the glass beads gaining a static charge as they were discharged from the hopper during a flow test. Researching this problem, I found that the solution is to electrically ground the hopper directly to a ground or metal surface, which solved the problem. It has also been a challenge acquiring all the material for this project, waiting more then a month for some of the items. Through this project’s problems and opportunities, I have gained much that I will take on with me as I continue the journey of becoming a chemical  engineer.

The projects goal is to make connections between the factors that affect the flow rate of glass beads through a hopper. These connections will then be used as the basis to extrapolate the trends found, to other particles of various shapes and sizes. This research can then be used to prevent common problems found in industry and make all processes that use hoppers to discharge particles more efficient.